Transfer Line Balancing Problem

Transfer Line Balancing Problem (TLBP) was introduced in (Dolgui et al., 2000). In comparison with the well-known assembly line balancing problems, TLBP has many new assumptions reflecting the particularities of the machining environment. Machining transfer lines are usually paced and serial. They consist of a sequence of stations linked by an automated material handling device. In lines of this type, each station is equipped by a special machine-tool which performs machining operations block by block. All operations of each block are executed simultaneously using one multi-spindle head. The parallel execution of operations is possible due to the fact that the multi-spindle heads carry several simultaneously activated tools. When machining at the current station is finished (all blocks installed on this machine have been activated) the part is moved to the next station. The time span between two movements can not exceed the given time value T0 referred to as line cycle time. The balancing problem consists in assigning the given set of operations to parallel blocks and stations under given assignment restrictions.

The objective is to configure a serial machining line that consists of a number of linearly ordered machines. All the machines are linked by an automated material handling device without buffers between machines. Each machine is equipped with multi-spindle heads activated sequentially by changing the active spindle head or by moving the part to successive spindle heads. These multi-spindle heads perform different machining operations on the manufactured part. This type of equipment allows performing simultaneously several operations by tools fixed in the spindle head and activated simultaneously. The part transport is synchronized: all parts of the line are transferred simultaneously to the next station.

The line balancing problem in the assembly environment is well-studied in the literature – see e.g. (Baybars, 1986). The TLBP has a number of unique characteristics such as parameterized operation times, non-strict precedence constraints, and parallel operations execution. These features make it impossible to use directly the optimization methods developed for the Assembly Line Balancing Problems. Several exact (e.g. mixed-integer programming and graph approaches) and heuristic (e.g. FSIC and multi-start decomposition algorithm) methods have been developed for the TLBP, a summarized description of these methods is given in (Guschinskaya, Dolgui, 2006). Later, in (Guschinskaya, Dolgui, 2007; Dolgui, Eremeev, Guschinskaya 2008) it was proposed to use greedy randomized adaptive search procedure (GRASP) and a genetic algorithm (GA) for solving this problem.

Notation

Let N (i Î N) be the set of all operations i required to machine a part. Then, a solution of TLBP can be represented by a collection P= {{N11,…, N1},…,{Nm1,…,Nm}} , determining an assignment of operations to a sequence of machines (k=1, 2, …, m) and repartition of operations assigned to the same machine k to nk blocks.  Let Nk be the work content (i.e. a set of operations) for machine k (k=1, 2, …, m) and Nkl be the set of operations grouped into common block l ( l=1, 2, …, nk) of machine k.

To distinguish the pieces of equipment and corresponding sets of operations, the following notions are used: “station” for a set of operations assigned to a machine and “block” for a set of operations assigned to a spindle head.

The following input data is assumed to be known about the set of operations:

- Precedence constraints between the operations. The technological process defines a partial order relation over operation set N. They can be represented by digraph G = (N, D). The arc (i, j) Î N´N belongs to set D if and only if operation j cannot precede operation i. Such precedence constraints are called non strict because if operation i is a predecessor of operation j then either operation i can be executed before j or i and j can be executed simultaneously using a compound tool;

- Inclusion constraints containing the groups of operations that must be assigned to the same machine, because of a required machining tolerance. They are can be represented by ES, a family of subsets from N, such that all operations of the same subset e Î ES must be assigned to the same station. No two sets from ES can include the same operation; otherwise such sets should be united.

- Station exclusion constraints containing the groups of operations that cannot be assigned to the same machine because of their technological incompatibility. They can be represented by , a family of subsets from N, such that each subset e Î  cannot be assigned to the same station as a whole.

- Block exclusion constraints containing the groups of operations that cannot be assigned to the same spindle head because of their technological incompatibility. They can be represented by , a family of subsets from N, such that the same subset e Î  cannot belong to the same block as a whole.

# The following input data describes the machinery that can be used in the line design:

- b is an auxiliary time needed for activation of a spindle head;

- S is an auxiliary time needed for loading/unloading the part on a machine;

- C1 is the relative cost of one station;

- C2 is the relative cost of one block;

- n0 is the maximal number of blocks per station;

The following data describes the line constraints to be respected:

- m0 is the maximal authorized number of stations; this parameter is introduced in order to reduce the number of feasible solutions. Its value can be obtained by analyzing the available factory area or the maximum authorized line investment cost, or calculated as an upper bound on m.

- T0 is the maximal admissible line cycle time (desired productivity).

To calculate the line cycle time that represents the time delay between the production of two products and to verify the line cycle constraint, the following calculation has to be made:

- For each operation j, its required working stroke length j and the maximal admissible feed per minute sj are given. The working stroke length includes the required depth of cut and the distance between the tool and the part surface.

- The block processing time tb(Nkl) is determined as follows:

tb(Nkl) = max{j | j  Nkl} / min{sj | j  Nkl } + b.

- Since the blocks of the same station are activated sequentially, the station processing time tS(Nk) is equal to the sum of its block processing times: tS(Nk) = + S.

Mixed integer program (MIP) for solving TLBP (Dolgui et. al, 2006a; Guschinskaya, Dolgui, 2006).

Model notations:

q is the index of block, e.g., for the block l of a station k, q=(k-1)n0 +l;

q0 is the maximal possible value of q, q0=m0n0;

S(k)={(k-1)n0 +1, …, kn0} is the set of block indices for a station k;

Q(j) is the set of indices q (blocks) where operation j can be assigned;

K(j) is the set of indices k (stations) where operation j can be assigned;

e is a set of operations which is an element of ES, ESor EB;

j(e) is some fixed operation from the set e.

tj =j/s j is the execution time of operation j if it is performed alone in a block.

tij = max{i , j } / min{ si,sj} is the execution time of two operations i,j if they are

performed in one block. In case sj are all identical, this value is not used.

Variables:

Xjq is a binary decision variable (1 if operation j is assigned to block q and 0 otherwise);

Fq ³ 0 is an auxiliary variable for determining the time of block q;

Yq  {0, 1} is an auxiliary variable that indicates if block q exists or not;

Zk  {0, 1} is an auxiliary variable that indicates if station k exists or not. The variables Yq and Zk are used to count the number of blocks and stations, respectively.

To reduce the number of decision variables and constraints, the intervals of possible values of block Q(j) and station K(j) numbers for each operation are obtained from the analysis of all the constraints (for details, see Dolgui et al., 2006b).

The objective function is represented by (1). The precedence constraints are formulated in (2). Constraints (3) reflect the fact that each operation must be assigned to exactly one block. Constraints (4) determine the necessity of grouping certain operations in the same station. Constraints (5)-(6) deal with the impossibility of grouping certain operations in one block or executing certain operations at the same station, respectively. Constraints (7) and (7’) determine the block processing times: here condition (7) corresponds to the case of single operation in block, while (7’) covers the cases of two or more operations. In case sj are all identical, inequality (7’) is redundant. Constraint (8) is the constraint on the cycle time. Constraints (9) ensure that block q exists in the design decision if and only if Xjq =1 at least for one j. Constraints (10) ensure that station k exists in the design decision if and only if Yq =1 at least for one qS(k). Constraints (11) guarantee that block q is created in station k only if block q-1 exists for this station. Constraints (12) ensure that station k can be created only if station k-1 exists at the line. Inequalities (11) and (12) mainly serve as symmetry-breaking cuts in this model (note that by a simple modification of (10) one could make these inequalities redundant). Bounds (13) are also imposed to reduce the polyhedron of linear relaxation.

Min W(Zk,Yq)= 		(1)

subject to:

, (i,j)ÎD		(2)

q∈Q(j)Xjq=1,  jÎN	(3)

eES, kK(j(e))	(4)

eEB, qintersectj∈e Q(j)	(5)

eES, k	(6)

Fq ³ ti Xiq, iÎN, qÎQ(i)	(7)

Fq ³ tij (Xiq + Xjq  - 1), i,j ÎN, i<j, qÎQ(i) Q(j)	(7’)

T0, k=1, 2,…, m0	(8)

Yq³Xjq, jÎN, qÎQ(j)	(9)

Zk ³ Yq, k=1, 2,…, m0, q=(k-1)n0+1	(10)

Yq-1 – Yq ³ 0, qÎS(k)\{(k-1)n0+1}, k= 1, 2,…, m0	(11)

Zk-1 – Zk ³ 0, k=2, 3,…, m0	(12)

Fq Î[0, T0 - s - b], q=1, 2,…, q0	(13)

This model in GAMS can be written as suggested below. The input data is imported from file “problem.gms”, formatted as the benchmarks of data sets S1-S7. The output is written in file “results.csv”.

$offsymxref

$offsymlist

$include "problem.GMS"

$setglobal CPU_TIME_CPLEX 90

* CPU time limit for CPLEX

$setglobal ABS_ERROR_CPLEX 0

* Absolute error limit for CPLEX

scalar r temporary value;

*========================================== calculated sets and parameters

set QQ(j,q) "set of indices q of blocks to which operation j can be assigned";

loop((j,q)$((ord(q) ge a(j)) and (ord(q) le b(j))), QQ(j,q)=yes);

set KK(j,k) "set of indices k of stations to which operation j can be assigned";

loop((j,k)$((ord(k) ge AA(j)) and (ord(k) le BB(j))), KK(j,k)=yes);

set SS(k,q) the set of block indices q for the stations k;

loop(k,

loop(q$((ord(q) > (ord(k)-1)*n_0) and (ord(q) le ord(k)*n_0)),

SS(k,q)= yes;

);

);

parameters

IS_size(IS_name) sizes of station inclusion subsets e,

XS_size(XS_name) sizes of station exclusion subsets e,

XB_size(XB_name) sizes of block exclusion subsets e

;

set je_IS(j,IS_name) je is some fixed operation from the set IS_name;

set KK_IS(k,IS_name) the set of stations where all operations of IS_name can be placed;

KK_IS(k,IS_name)=yes;

loop(IS_name,

r=smin(IS(IS_name,j),ord(j));

* choose the operation with minimal index in IS_name as je_IS for IS_name

loop(j$(ord(j)=r), je_IS(j,IS_name)=yes);

IS_size(IS_name)=sum(IS(IS_name,j),1);

loop((j,k)$(IS(IS_name,j) and not(KK(j,k))),

KK_IS(k,IS_name)=no;

)

);

set je_XS(j,XS_name) je is some fixed operation from the set XS_name;

loop(XS_name,

r=smin(XS(XS_name,j),ord(j));

* choose the operation with minimal index in XS_name as je_XS for XS_name

loop(j$(ord(j)=r), je_XS(j,XS_name)=yes);

XS_size(XS_name)=sum(XS(XS_name,j),1);

);

set je_XB(j,XB_name) je is some fixed operation from the set XB_name;

loop(XB_name,

r=smin(XB(XB_name,j),ord(j));

* choose the operation with minimal index in XB_name as je_XB for XB_name

loop(j$(ord(j)=r), je_XB(j,XB_name)=yes);

XB_size(XB_name)=sum(XB(XB_name,j),1);

);

parameter

t(i,j) supplementary parameter to compute the block time t_b

;

loop((i,j),

t(i,j)=max(l(i),l(j))/min(s(i),s(j));

);

display t;

parameter

min_num_st "minimal number of stations (computed on the basis of AA)"

;

min_num_st=smax(j,AA(j));

scalar UseEq7Prime the flag to use (7') if not all s_j are identical;

UseEq7Prime=0;

loop(j$(ord(j)>1),

if(not(s(j)=s(j-1)),

UseEq7Prime=1;

);

);

*======================================== end of calculated sets and parameters

alias (q, qprime);

alias (j,je);

alias (q,qminus1);

alias (k,kminus1);

*======================================== Model

Binary Variables

Z(k) "indicates if the station k exists or not"

Y(q) "indicates if the block q exists or not"

;

Positive Variables

F(q) "the time of block q"

;

Binary Variables

X(j,q) "decision variable: equals to 1 iff operation j is assigned to block q"

Variables

cost "the overall cost, to be minimized"

;

Equations

obj

cone_block_op(j)

cpreced2(i,j)

cincusion_st(IS_name,k,je)

cexclusion_bl(XB_name,q)

cexclusion_st(XS_name,k)

clb_block_time1(i,q)

clb_block_time2(i,j,q)

cub_station_time(k)

cexist_block(q,j)

cexist_station(q,k)

ccont_block(q,k,qminus1)

ccont_station(k,kminus1)

cut_station(k)

cut_block(q)

;

* (1) Objective function (minimization)

obj..

cost =e= C_1 * sum(k, Z(k)) + C_2 * sum(q, Y(q));

* (2") Precedence constraint

cpreced2(i,j)$(Pred(i,j))..

sum(q$QQ(j,q), (ord(q)-1)*X(j,q)) =g= sum(q$QQ(i,q), (ord(q)-1)*X(i,q));

* (3) Each operation is assigned only to one block

cone_block_op(j)..

sum(q$(QQ(j,q)), X(j,q))=e= 1;

* (4) Inclusion constraint for stations

cincusion_st(IS_name,k,je)$(KK(je,k) and je_IS(je,IS_name))..

sum(j$(IS(IS_name,j) and not je_IS(j,IS_name)),

sum(q$(SS(k,q) and QQ(j,q)), X(j,q))

)

=e=(IS_size(IS_name)-1) * sum(q$SS(k,q), X(je,q));

* (5) Exclusion constraint for blocks

cexclusion_bl(XB_name,q)$

( sum(j$(XB(XB_name,j) and QQ(j,q)),1) = XB_size(XB_name) )..

sum(j$XB(XB_name,j), X(j,q)) =l= XB_size(XB_name)-1;

* (6) Exclusion constraint for stations

cexclusion_st(XS_name,k)$

( sum(j$(XS(XS_name,j) and KK(j,k)),1) = XS_size(XS_name) )..

sum(j$XS(XS_name,j),

sum(q$(SS(k,q) and QQ(j,q)),

X(j,q)

)

) =l= XS_size(XS_name)-1;

* (7) lower bound on block time

clb_block_time1(i,q)$(QQ(i,q))..

F(q) =g=t(i,i)* X(i,q);

* (7') lower bound on block time

clb_block_time2(i,j,q)$(ord(i)<ord(j) and QQ(i,q) and QQ(j,q))..

F(q) =g= t(i,j)*(X(i,q)+X(j,q)-1) - (1-UseEq7Prime)*T_0;

* (8) the station time does not exceed the cycle time T_0

cub_station_time(k)..

tau_s+sum(q$SS(k,q), F(q)) =l= T_0;

* (9) existence of block q

cexist_block(q,j)$QQ(j,q)..

Y(q) =g= X(j,q);

* (10) existence of station k

cexist_station(q,k)$(ord(q)=(ord(k)-1)*n_0+1)..

Z(k) =e= Y(q);

* (11) block q can be created only if block q-1 exists in station k

ccont_block(q,k,qminus1)$(SS(k,q) and not(ord(q)=(ord(k)-1)*n_0+1) and ord(qminus1)=ord(q)-1)..

Y(qminus1) - Y(q) =g= 0;

* (12) staion k can be created only if station k-1 exists

ccont_station(k,kminus1)$(ord(k)>1 and ord(kminus1)=ord(k)-1)..

Z(kminus1) - Z(k)=g= 0;

* Additional constraint that serves as a cut

cut_station(k)..

Z(k) =l= sum(q$SS(k,q), Y(q));

* Additional constraint that serves as a cut

cut_block(q)..

Y(q) =l= sum(j$QQ(j,q), X(j,q));

*Bounds on variables

Y.up(q)= 1;

Z.up(k)= 1;

Y.lo(q)= 0;

Z.lo(k)= 0;

F.up(q)=T_0-tau_s-tau_b;

* Eliminate/fix unnecesary binary variables

loop((j,q)$(not(QQ(j,q))), X.up(j,q)=0);

loop((j,k)$(not(KK(j,k))),

loop(q$SS(k,q),X.up(j,q)=0);

);

loop(k$(ord(k) le min_num_st),

Z.fx(k)=1;

loop(q$(ord(q) = (ord(k)-1)*n_0+1), Y.fx(q)=1);

);

Model MIP2

/

obj

cone_block_op

cpreced2

cincusion_st

cexclusion_bl

cexclusion_st

clb_block_time1

clb_block_time2

cub_station_time

cexist_block

cexist_station

ccont_block

ccont_station

cut_station

cut_block

/;

MIP2.NodLim = 1e09;

MIP2.holdfixed = 1;

option mip = CPLEX;

option optcr = 0.0;

option optca = %ABS_ERROR_CPLEX%;

option reslim = %CPU_TIME_CPLEX%;

option iterlim = 1e08;

option solprint = off;

file series_report / results.csv /;

series_report.ap=1;

put series_report;

put " ";

Solve MIP2 using MIP minimizing cost;

put series_report;

put "f best found  , CPLEX CPU time (sec), CPU time given (sec) , modelstat flag;

put solvestat flag, variables, LB, Use (7')? " /;

if(not(execerror) and (MIP2.modelstat=1  or MIP2.modelstat=8 ),

put cost.l, " , ", MIP2.resusd, " , ", %CPU_TIME_CPLEX% , " , ",MIP2.modelstat;

put " , ", MIP2.solvestat , " , ", MIP2.numvar , " , ", MIP2.objest, ", ",UseEq7Prime;

else

put "NOT FOUND", " , ", " , ", %CPU_TIME_CPLEX% , " , ", MIP2.modelstat;

put " , ", MIP2.solvestat , " , ", MIP2.numvar, ", ", " , ", UseEq7Prime;

);

display X.l;

REFERENCES

Baybars, I., 1986. A survey of exact algorithms for the simple assembly line balancing. Management Science 32, 909–932.

Dolgui, A., Eremeev A, Guschinskaya, 2008. O. MIP-based GRASP and Genetic algorithm for balancing transfer lines. Submitted to Proc. Matheuristics Workshop, June 16-18, 2008, Bertinoro, Italy., 22p.

Dolgui, A., Guschinsky, Levin, G., 2000. Approaches to balancing of transfer lines with blocks of parallel operations; Preprints No 8, Institute of Engineering Cybernetics of Academy of Sciences of Belarus/University of Technology of Troyes, 42p.

Dolgui, A., Finel, B., Guschinsky, N., Levin, G. and Vernadat, F., 2006a. MIP approach to balancing transfer lines with blocks of parallel operations, IIE Transactions 38, 869–882.

Dolgui, A., Guschinsky, N., Levin, G. , 2006b A special case of transfer lines balancing by graph approach, European Journal of Operational Research 168 (3), 732-746.

Guschinskaya, O., Dolgui, A., 2006. A Comparative Evaluation of Exact and Heuristic Methods for Transfer Line Balancing Problem, in: Information Control Problems In Manufacturing 2006: A Proceedings volume from the 12th IFAC International Symposium, St Etienne, France, 17-19 May 2006, A. Dolgui, G. Morel, C. Pereira, eds., Elsevier Science, ISBN: 978-0-08-044654-7, Vol. 2, p. 395–400.

Guschinskaya, O., Dolgui A., 2007. Balancing transfer lines with multiple-spindle machines using GRASP. Unpublished manuscript.