Series S1-S5 (Guschinskaya, Dolgui, 2006)

Each of the series S1-S5 contains 50 transfer line balancing problems. These data sets differ by two parameters: the number of operations |N| and the number of constraints (order strength OS of the precedence graph, number of subsets in families ES, andES). Note: the order strength is defined as the number of edged in the transitive closure of the precedence graph divided by (|N|-1)∙|N|/2  (Scholl and Klein, 1998). Each instance was generated at random. The operation times are derived from the uniform distribution within [10, 20]. The precedence graph was generated at random by adding arcs between randomly chosen vertexes until desired order strength is obtained. The operations related by compatibility constraints were chosen at random but with the aim to avoid contradicting constraints and assure existence of a feasible solution. Other input parameters were set as follows: n0=4, b =0, S=0, T0=100, sj=1 for all operations j, C1=10, and C2=2. The parameter m0 varied for the data sets with different numbers of operations.

Small size problems of series S1 and S2. The first data set contains 25-operation problems with OS= 0.5, m0 = 5. The second data set contains 25-operation problems with OS= 0.15, |ES| = 1, m0 =4. All of these problems are solved to optimality (Guschinskaya, Dolgui, 2006).

| Series S1 |  | Series S2 |  |
| --- | --- | --- | --- |
| Problem | Optimum | Problem | Optimum |
| 1 | 38 | 1 | 28 |
| 2 | 38 | 2 | 36 |
| 3 | 38 | 3 | 36 |
| 4 | 48 | 4 | 48 |
| 5 | 60 | 5 | 40 |
| 6 | 40 | 6 | 38 |
| 7 | 38 | 7 | 36 |
| 8 | 38 | 8 | 36 |
| 9 | 38 | 9 | 38 |
| 10 | 50 | 10 | 38 |
| 11 | 54 | 11 | 36 |
| 12 | 48 | 12 | 38 |
| 13 | 40 | 13 | 38 |
| 14 | 48 | 14 | 40 |
| 15 | 40 | 15 | 38 |
| 16 | 42 | 16 | 30 |
| 17 | 38 | 17 | 36 |
| 18 | 48 | 18 | 38 |
| 19 | 38 | 19 | 36 |
| 20 | 48 | 20 | 38 |
| 21 | 42 | 21 | 28 |
| 22 | 50 | 22 | 48 |
| 23 | 38 | 23 | 38 |
| 24 | 38 | 24 | 36 |
| 25 | 48 | 25 | 36 |
| 26 | 42 | 26 | 38 |
| 27 | 60 | 27 | 38 |
| 28 | 48 | 28 | 48 |
| 29 | 38 | 29 | 28 |
| 30 | 30 | 30 | 28 |
| 31 | 38 | 31 | 28 |
| 32 | 50 | 32 | 48 |
| 33 | 38 | 33 | 36 |
| 34 | 38 | 34 | 36 |
| 35 | 38 | 35 | 36 |
| 36 | 50 | 36 | 48 |
| 37 | 42 | 37 | 40 |
| 38 | 60 | 38 | 36 |
| 39 | 52 | 39 | 38 |
| 40 | 38 | 40 | 28 |
| 41 | 38 | 41 | 36 |
| 42 | 38 | 42 | 38 |
| 43 | 50 | 43 | 28 |
| 44 | 38 | 44 | 38 |
| 45 | 36 | 45 | 36 |
| 46 | 38 | 46 | 38 |
| 47 | 48 | 47 | 38 |
| 48 | 48 | 48 | 38 |
| 49 | 48 | 49 | 38 |
| 50 | 38 | 50 | 36 |

Medium size problems of series S3 and S4. The data set S3 contains 50-operation problems with OS= 0.9, m0 = 10.  The data set S4 contains 50-operation problems with OS= 0.45, m0 = 15.   All of these problems in S3 are solved to optimality (Guschinskaya, Dolgui, 2006) as well as some problems in S4, for the rest of the instances in S4 the heuristic solutions (Dolgui, Eremeev, Guschinskaya 2008) provide the upper bounds.

| S3 |  | S4 |  |  |
| --- | --- | --- | --- | --- |
| Problem | Optimum | Problem | Optimum | Upper Bound |
| 1 | 112 | 1 | 76 | 76 |
| 2 | 114 | 2 |  | 102 |
| 3 | 100 | 3 |  | 68 |
| 4 | 124 | 4 |  | 74 |
| 5 | 118 | 5 | 76 | 76 |
| 6 | 82 | 6 |  | 66 |
| 7 | 106 | 7 |  | 78 |
| 8 | 94 | 8 |  | 74 |
| 9 | 88 | 9 |  | 76 |
| 10 | 102 | 10 | 74 | 74 |
| 11 | 110 | 11 |  | 86 |
| 12 | 82 | 12 | 68 | 68 |
| 13 | 100 | 13 | 76 | 76 |
| 14 | 92 | 14 |  | 78 |
| 15 | 78 | 15 |  | 76 |
| 16 | 100 | 16 | 66 | 66 |
| 17 | 100 | 17 | 78 | 78 |
| 18 | 88 | 18 |  | 96 |
| 19 | 104 | 19 | 76 | 76 |
| 20 | 102 | 20 | 78 | 78 |
| 21 | 92 | 21 | 76 | 76 |
| 22 | 88 | 22 | 66 | 66 |
| 23 | 100 | 23 | 74 | 74 |
| 24 | 102 | 24 |  | 64 |
| 25 | 104 | 25 |  | 76 |
| 26 | 104 | 26 | 78 | 78 |
| 27 | 80 | 27 | 64 | 64 |
| 28 | 102 | 28 |  | 62 |
| 29 | 96 | 29 | 74 | 74 |
| 30 | 112 | 30 |  | 76 |
| 31 | 112 | 31 | 76 | 76 |
| 32 | 82 | 32 |  | 54 |
| 33 | 104 | 33 | 74 | 74 |
| 34 | 112 | 34 |  | 62 |
| 35 | 100 | 35 | 76 | 76 |
| 36 | 112 | 36 | 76 | 76 |
| 37 | 106 | 37 | 76 | 76 |
| 38 | 102 | 38 | 66 | 66 |
| 39 | 116 | 39 |  | 102 |
| 40 | 108 | 40 |  | 76 |
| 41 | 92 | 41 | 86 | 86 |
| 42 | 114 | 42 | 76 | 76 |
| 43 | 108 | 43 |  | 68 |
| 44 | 114 | 44 | 74 | 74 |
| 45 | 106 | 45 | 64 | 64 |
| 46 | 90 | 46 | 64 | 64 |
| 47 | 100 | 47 |  | 76 |
| 48 | 112 | 48 |  | 86 |
| 49 | 76 | 49 | 74 | 74 |
| 50 | 110 | 50 | 98 | 98 |

Large size problems of series S5. The data set contains 100-operation problems with OS= 0.25, m0 =15. The heuristic solutions (Dolgui, Eremeev, Guschinskaya 2008) provide the upper bounds.

| Series S5 |  |
| --- | --- |
| Problem | Upper bound |
| 1 | 106 |
| 2 | 72 |
| 3 | 78 |
| 4 | 78 |
| 5 | 102 |
| 6 | 70 |
| 7 | 96 |
| 8 | 68 |
| 9 | 90 |
| 10 | 82 |
| 11 | 70 |
| 12 | 108 |
| 13 | 94 |
| 14 | 78 |
| 15 | 76 |
| 16 | 70 |
| 17 | 82 |
| 18 | 80 |
| 19 | 94 |
| 20 | 80 |
| 21 | 92 |
| 22 | 84 |
| 23 | 82 |
| 24 | 82 |
| 25 | 68 |
| 26 | 90 |
| 27 | 68 |
| 28 | 68 |
| 29 | 104 |
| 30 | 108 |
| 31 | 82 |
| 32 | 78 |
| 33 | 86 |
| 34 | 96 |
| 35 | 72 |
| 36 | 90 |
| 37 | 84 |
| 38 | 94 |
| 39 | 88 |
| 40 | 104 |
| 41 | 88 |
| 42 | 84 |
| 43 | 104 |
| 44 | 80 |
| 45 | 106 |
| 46 | 90 |
| 47 | 102 |
| 48 | 92 |
| 49 | 78 |
| 50 | 68 |

Series S6-S7 (Guschinskaya, 2007)

Series S6 and S7 are also randomly generated but with more realistic data sets, compared to S1-S5. Both series S6 and S7 consist of 20 instances, here C1=1, C2=0.5, n0=4, b =0.2, S=0.4. The number of operations |N| for series S6 is from 46 to 92, and for series S7 it is from 94 to 125. The upper bounds m0 on the number of stations are from 23 to 46 in series S6 and from 43 to 62 in S7. The details on random generation of series S6, S7 can be found in (Guschinskaya, 2008).

The series S6 and S7 consist of realistic instances for two reasons. Firstly, they contain non-trivial input data for parameters with various sj. Secondly, in S6 and S7 the pseudo-random choice of operations was not performed independently and uniformly as in series S1-S5, but based on real-life data of typical shapes of the parts manufactured in mechanical transfer lines. The random choice is applied to the shape of parts, which further defines the parameters and mutual compatibility of operations. The heuristic solutions (Dolgui, Eremeev, Guschinskaya 2008) provide the upper bounds.

| Series S6 |  | Series S7 |  |
| --- | --- | --- | --- |
| Problem | Upper bound | Problem | Upper bound |
| 1 | 28 | 1 | 40.5 |
| 2 | 32 | 2 | 67 |
| 3 | 47.5 | 3 | 50 |
| 4 | 46 | 4 | 58.5 |
| 5 | 27 | 5 | 50.5 |
| 6 | 34.5 | 6 | 50.5 |
| 7 | 38.5 | 7 | 44.5 |
| 8 | 27.5 | 8 | 46 |
| 9 | 26 | 9 | 35.5 |
| 10 | 42 | 10 | 66 |
| 11 | 36 | 11 | 38 |
| 12 | 45.5 | 12 | 36 |
| 13 | 45 | 13 | 50 |
| 14 | 38 | 14 | 47.5 |
| 15 | 38.5 | 15 | 38 |
| 16 | 29 | 16 | 45.5 |
| 17 | 43.5 | 17 | 49.5 |
| 18 | 43.5 | 18 | 39 |
| 19 | 44 | 19 | 43 |
| 20 | 31.5 | 20 | 55.5 |

Additional information given in the problem files

The intervals of possible values of block Q(j) and station K(j) numbers for each operation j are given in terms of their lower and upper bounds: a(j) is lower bound on block number of operation j, b(j) is the upper bound on block number of operation j, AA(j) is the lower bound on minimal station number of operation j, BB(j) is the upper bound on maximal station number of operation j. These bounds are obtained from the analysis of all the constraints (for details, see Dolgui et al., 2006).

REFERENCES

Dolgui, A., Eremeev A, Guschinskaya, 2008. O. MIP-based GRASP and Genetic algorithm for balancing transfer lines. Submitted to Proc. Matheuristics Workshop, June 16-18, 2008, Bertinoro, Italy., 22p.

Guschinskaya, O., 2007. Outils d'aide a la decision pour la conception en avant-projet des systemes d'usinage a boitiers multibroches.} PhD thesis, Saint-Etienne, France.

Guschinskaya, O., Dolgui, A., 2006. A Comparative Evaluation of Exact and Heuristic Methods for Transfer Line Balancing Problem, in: Information Control Problems In Manufacturing 2006: A Proceedings volume from the 12th IFAC International Symposium, St Etienne, France, 17-19 May 2006, A. Dolgui, G. Morel, C. Pereira, eds., Elsevier Science, ISBN: 978-0-08-044654-7, Vol. 2, p. 395–400.

Dolgui A., Finel, B., Guschinsky, N., Levin, G. and Vernadat, F., 2006b. MIP approach to balancing transfer lines with blocks of parallel operations, IIE Transactions 38, 869–882.